—

# Project Pitch: Automatic Plant Watering System

Group Member Names: Isaac Mangano, Nick Martini, Matus Trinka

Target Specialisation: Smart Agriculture

Hardware Platform: ESP32

## 1. Executive Summary

Plants require suitable environmental conditions to remain healthy such as water and light. Monitoring plants can be difficult especially for people who are busy, away from home a lot or unsure how much water do plants require. This can result in plants being under-watered.

This project aims to develop an IoT-based smart plant monitoring and automatic watering system that monitors soil moisture and environmental conditions. It will also automatically water the plant when required and alert the user when there is insufficient water available.

## 2. System Architecture

- Sensing (Inputs)

  - Capacative Moisture Sensor: This sensor is placed in the soil and sends an analog signal to the ESP32

  - Ultrasonic Distance Sensor: This sensor is placed inside the water tank to detect distance from the water. A higher distance reading will mean that the water levels is lower. Thresholds will be calculated with this distance.

- Processing (Thinking)

    - Core Logic: An ESP32 based FSM (finite state machine)

    - Intelligence: Uses inputs from soil moisture to enter one of the following states:

      - IDLE (The ESP32 sleeps and waits until the next soil moisture check)

      - WATERING (The ESP32 has the pump relay enabled while the soil moisture is below a threshold)

- IDLE_HOT_DAY ( Same as IDLE but checking the soil moisture on more regular intervals)

      - WATER_EMPTY (The ESP32 triggers the RED_LED_1 and Buzzes, re-checks water levels in regular intervals)

      - ERROR (The edge intelligence has noticed a broken sensor, RED_LED_2 is turned on and the device sleeps indefinitely as the sensors need replacing)

- Actuation (Outputs)

- Green LED: Triggered on IDLE and WATERING states

- Yellow LED: Triggered on the IDLE_HOT_DAY state to warn users of high expected water usage

- Red LED 1: Triggered on WATER_EMPTY to notify user to add water to the tank

- Red LED 2: Triggered on ERROR to notify user to check and replace sensors

- Buzzer: Activated on WATER_EMPTY and ERROR states to notify users of a necessary user action

- Water pump relay: activated on WATERING state to trigger the water pump with sufficent voltage and current that the ESP32 can’t provide

## 3. Behavioural Logic

| State | Trigger Condition | Action / Output |
| --- | --- | --- |
| IDLE | Initial state. Triggered from WATERING state when moisture exceeds threshold. | - Green LED on - Pump off - ESP32 sleeps and rechecks condition every 2 hours |
| WATERING | Triggered from IDLE state when soil moisture is below 35% | - Pump on |
| IDLE_HOT_DAY | Triggered from IDLE state when weather API detects heat above 30 degrees celcius | - Yellow LED on - Pump off - ESP32 sleeps and rechecks condition every hour |
| WATER_EMPTY | Triggered form IDLE or IDLE_HOT_DAY states when the water level is below 5% | - Red LED (1) on - Buzzer on for 5 seconds - ESP32 sleeps in 30 minute intervals and rechecks the condition |
| ERROR | Triggered from all other states when Edge AI detects unusual sensor behaviour | - Red LED (2) on - Buzzer on for 5 seconds - ESP32 sleeps indefinitely as user needs to replace sensors and reboot the ESP32 |

## 4. IoT Communication

Sensors→ESP32→Wi-Fi→MQTT→Cloud platform→dashboard

The ESP32 will use Wi-Fi and MQTT to share the sensor data to the cloud. Then it will be displayed on a dashboard which will allow the user to remotely check the plant’s condition and see the historical data.

## 5. Cloud Integration

Sensors→ESP32→Wi-Fi→MQTT→Adafruit IO→dashboard

Adafruit IO will be used by the system as the cloud platform.

## 6. Edge Intelligence

We will use a rule based intelligence to detect unusual sensor behaviour. This will notify the ESP32 and trigger the ERROR state. This indicates that the user must replace one or more sensors.

## 7. Security

When connecting the ESP32 to Adafruit IO, the user will enter username and API key which will be used to authenticate the user.